Sierra College                MECH-01

Jim Weir              530.272.2203

jweir@sierracollege.edu

www.rstengineering.com/sierra

Application 10

- Compiling, Programming, and Executing

- Compiling the Program

- Open Microcode Studio.

- Open a New program file.  Take the preset header (with all the asterisks (*) around it off.

- Cut and paste  the program from “Theory” into Studio  (see next page).

- Rename the program something you will remember

- Save the program anywhere you like on the hard disk, but SAVE IT TO YOUR FLASH DRIVE as well.  Be sure to save it as a .bas file and not a .pgp or other suffix file.  Note that at the end of class, the hard disk copy will be erased.

- Exit Studio

- Reopen studio and open your program FROM YOUR FLASH DRIVE

- Look the program over to be sure that it is the one you “wrote” or pasted.

- Compile the program with the Compile Program command.

- If the program shows errors, fix the errors.  Otherwise, the program should show “Compile Successful No Errors.

- Your flash drive should have the following files on it (this is mine, yours should have your initials on it):

- Your hex file will look something like this:

:020000040000FA

:10000000012883160830850087018312071683169E

:0E001000071283128A01062863008A010C2859

:02400E00FC3F75

:00000001FF

- Take your BASIC  .bas file and your Compiled .hex file to the instructor for programming.

.

- Using a trainer, construct the circuit in “LED Test” below.  You may use a dummy or scrap 14 pin  to lay out the connections before installing the actual microcontroller chip.  Test the LED, and then see if pin 6 of the microcontroller lights the light.  Put the free end of the resistor on any pin but 1 and 6 to see if any of the other pins are HIGH (+5 volts).  No other pin should light the LED

.