Sierra College                MECH-01

Jim Weir              530.272.2203

jweir@sierracollege.edu

www.rstengineering.com/sierra

Application 12

- Preparing the switch

- The first thing we have to do is to prepare a switch so that we can use it to control the 688.  Notice that the switch has six sides.  One side has a pushbutton.  The opposite side is totally blank.

.

- The four small sides have terminals on two of the sides and nothing on the other small sides.

- Cut the two terminals off on one of the sides that have terminals.

- Solder small short wires to the remaining terminals on the remaining side.

- Connect this switch, one lead to ground (0 volts) and one lead to pin 8 of the 688.

- Connect a 1k ohm resistor from pin 8 to +5

.

- Load hex file pushbutn.hex into your 688 at the instructor’s station.

- If you do NOT push the button, does BLINK come on?

- If you push the button, does BLINK come on and repeat forever?

- Write and load program IFTHEN into your microprocessor.

- Does the switch control when the program loads and executes?

- Write and load program FORNEXT into your microprocessor

- Does the program execute 10 times and then return to no lights?

- Write and load program GOSUB into your microprocessor.

- Connect a 220 ohm resistor in series with an LED in accordance with the schematic below.

- Does the “nanny light” come on between pushbutton activation?